;Switches ToLed's

;***************************************************************************

.include "m32def.inc"

;***** Global register variables

.def	temp 	=R20

.def	temp2 	=R23

reset:						;Main program entry point on reset

ldi	temp,0b11111111 	;PB0,1,2,3 outputs, PB4,5,6,7 outputs (STK500 LEDs)

out	DDRB,temp

ldi	temp,0b11111111	;write all STK500 LEDs OFF

out	PORTB,temp

ldi	temp,0b00000000 	;PA0,1,2,3 inputs, PA4,5,6,7 inputs (STK500 LEDs)

out	DDRA,temp

ldi	temp,0b11111111	;enable pull up resistor on each input pin of PORTA

out	PORTA,temp

forever:

in	temp2, PINA

out	PORTB,temp2

rjmp	forever