Να εξηγήσετε τι κάνει ο κώδικας VHDL ο οποίος μέσω ψηφιακής θύρας απο το Arduino εισάγει ψηφιακή πληροφορία.

library ieee;

use ieee.std_logic_1164.all;

entity arduino is

port(

clock: in std_logic; -- "virtual" clock from ard

input: in std_logic; -- sig from ard

reset: in std_logic; -- reset also button

button: in std_logic; -- trigger sig_to_ard

sig_to_ard: out std_logic;

fpga_led: out std_logic_vector(4 downto 0)

);

end arduino;

architecture arch of arduino is

signal temp: std_logic_vector(6 downto 0);

-- temp(6) -> start

-- temp(5..2) -> information

-- temp(1) -> parity bit

-- temp(0) -> stop

begin

process (clock, reset, temp)

begin

-- Reset whenever the reset signal goes low, regardless of the clock

if (reset = '0') then

temp <= "0000000";

fpga_led <= "11111";

-- If not resetting, update the register output on the clock's falling edge

elsif (falling_edge(clock) and temp(6) = '0') then

temp(6 downto 1) <= temp(5 downto 0);

temp(0) <= input;

end if;

-- process input information

if(temp(6) = '1') then

fpga_led <= not temp(5 downto 1);

end if;

end process;

sig_to_ard <= not button;

end arch;