CYBER management for a ship AND/ or a fleet of vessels.

Christos Vasilokostas

University of Thessaly, Computer Science Sector

Attica, Athens

Jan 2018

Christos Vasilokostas

Abstract

Marine sector nowadays are becoming increasingly complex and dependent on the extensive apply of digital and communications technologies throughout their operational life. Ships rely most in operation of maritime information technology (IT), operational technology (OT) and communications systems. The safety of the ship as well as the environmental and commercial consequences of not being prepared for a cyber incident may be significant. There is needed an appropriate management framework that can be used to reduce the risk of cyber incidents that could affect the safety and security of the ship, its personnel and the cargo.

Introduction

If a cyber attack to our personal computer can lead to enormous problems to our bank accounts and private life, imagine what catastrophic consequences could cause to ship’s operation. It is an undeniable fact that technology age has brought enormous benefits in marine sector and as technologies developing continuously, ships are increasingly use systems that rely on digitization, integration and automation, which calls for cyber risk management on board. As technology continues to develop, information technology (IT) and operational technology (OT) onboard ships are being connected together in the same network and more times there is a need to be connected to the Internet worldwide in order to be up to date or upgrade. This connection could bring the risk of unauthorized access or malicious attacks to the networks and profoundly in the ship’s systems. Risks are also possible to occur from authorized personnel, which without having “bad aim” access systems on board using flash disks or other removable devices, the known ‘’Bring your own device (BYOD)’’, so they could introduce malware or any other virus to that unconnected systems. Policies and procedures adjust to companies and personnel’s needs should address their control, use, and how to protect ship’s vulnerable data.

# Determination of vulnerability

The growing complexity of ships, and their need to connect with services provided from shore side networks through the Internet, makes onboard systems increasingly exposed to cyber attacks. This fact makes ship’s systems to be vulnerable either as a way to deliver a cyber attack, or as a system affected because of a successful cyber attack.

Its important to understand how risky is to connect shipboard systems to uncontrolled networks. If this happens, the human factor should be taken into consideration, as well as the most of cyber incidents are caused by personnel actions or inactions. As illustrated in Fig.1 ship is a complex cyber-physical engineered system that comprises six (6) vital asset types:

a. Plant and machinery, which include:

(1). Engine governor and Power management

(2).  Alarm system

(3).  Integrated control system

(4). Emergency response system.

b. Cargo management systems, which include:

(1).Cargo Control Room (CCR) and its equipment

(2). Level Indication System

(3). Valve Remote Control System

(4).Water Ingress Alarm System and Ballast Water      Systems

(5). Gas liquefaction.

c. Passenger servicing and management systems

(1). Property Management System (PMS)

(2). Ship passenger/seafarer boarding access systems

(3).Infrastructure support systems like Domain Naming System (DNS) and user authentication systems.

d. Access control systems

(1). Surveillance systems such as CCTV network

(2). Shipboard Security Alarm Systems (SSAS)

(3). Electronic “personnel-on-board” systems.

e. Communication systems, which include:

(1). Satellite communication equipment

(2).Voice Over Internet Protocols (VOIP) equipment

(3). Wireless networks (WLANs)

(4). Public address and general alarm systems

f. Navigation systems, which include:

(1). Positioning systems (GPS)

(2). Electronic Chart Display Information System (ECDIS)

(3).Dynamic Positioning (DP) systems

(4).Automatic Identification System (AIS)

(5).Global Maritime Distress and Safety System (GMDSS)

(6). Radar equipment

(7).Voyage Data Recorders (VDRs)

which, are used to provide a range of operational services.

The loss, or compromise, of one or more of these assets has the potential to impact upon:

a. The health and safety of staff and other people impacted upon by the work activities being undertaken and to whom a duty of care is owed.

b. Ship’s ability to operate safely and to not endanger other ships, maritime structures or the environment.

c.   The speed and efficiency at which the ship can operate. A virus affecting onboard systems can cause costly delays in getting to port.

d.    The business-sensitive information and resulting risk of blackmail.

e.   The reputation – corporate image. This can be tarnished if a company is seen as being vulnerable to hackers.

f.     The privacy personal data protection.

## Fig 1. ship is a complex cyber-physical engineered system

# Common vulnerabilities

There is a list of common cyber vulnerabilities, which may be found onboard ships:

a.  Obsolete and unsupported operating systems.

b.  Outdated or missing antivirus software.

c. Insufficient security configurations, including ineffective network management.

d. Computer networks, which lack boundary protection measures and there is not a segmentation of networks.

e.  Safety vital systems that are always connected with the shore side and not only in cases that system needs to be up to date.

d. Insufficient access controls for third parties including contractors and service providers.

# Cyber risk assessment

The purpose of the cyber risk assessment of a ship’s network and its systems and devices is to identify these vulnerabilities that could result in loss of confidentiality, integrity, loss of operation of the equipment and system, loss of network, or even affect the safe operation of a ship which can lead to dangerous circumstances. These vulnerabilities and weaknesses could be categorized to the following categories:

a.   Technical, such as software defects or outdated or unpatched systems.

b.     Design, such as access control procedures,

c.  Implementation errors such as misconfigured firewalls.

d.     Procedural or other user errors.

`	It is critical that during a cyber risk assessment there should be a review to the configuration of all computers, servers, routers, and cyber security technologies including firewalls.

# Cyber attack categories

There are two particular kinds of attack:

a.  Untargeted attacks, where a ship’s systems and data are not the specific target of a hacker, but consist one of a group of potential targets.

b.  Targeted attacks, where a ship’s systems and data are the specific target.

In the first targets, the untargeted ones, hackers can use tools and techniques with which can locate known vulnerabilities onboard a ship. There are listed some examples of tools and techniques:

a. Social engineering. Using social interaction via social media, an intruder can break security procedures in order to manipulate insider personnel.

b.  Phishing. Sending emails to a large number of “clients” requesting from them to visit a fake website in order to be informed for a shop sale or any other commercial information using a hyperlink included in the email, intruder can “mine” from them personal and other important information.

c. Water holing. Establishing a fake website in order to exploit visitors for example a site refer to sports.

d. Scanning. Attacking to large group of Internet insiders at random.

In the second targets, the targeted ones, intruders use more sophisticated tools and techniques specifically modified in order to attack to a particular ship. There are listed some examples of tools and techniques for these specific circumstances:

a. Spear-phishing. There is an alike method to the phishing but here individuals are targeted with personal emails, which contain malicious software.

b. Deploying botnets. They are used in order to deliver Distributed Denial of Service attacks.

c. Subverting the supply chain. Attacking a company or ship by compromising equipment or software which will be delivered to the company or ship.

# Stages of a cyber attack

The appropriate time is needed to prepare a cyber attack, it depends by the objectives and the purpose of the attacker as well as it depends on the resilience and the implemented security plan of the systems onboard a ship. The four stages of an attack could be:

a. Survey/Reconnaissance. Information that is found on Internet sites of a ship’s company, which is known as “open resources” can be used so as a potential hacker to be prepared for a cyber attack.

b. Delivery. Attackers may attempt to access company and ship systems and data via Internet connection through Company online services, including cargo tracking systems or Sending emails containing malicious software.

c. Breach. Depending on the significance of the breach, a hacker will be able to affect the ship’s systems’ operation, for example an attacker may be able to interrupt and change the display of chart data held on ECDIS or take the full control of a machinery management system and inactive the alarm system.

d.  Affect. A hacker will explore systems and expand access to systems in order to access commercially sensitive data about cargo and personnel, manipulate crew and cargo manifests so as to allow the transport of illegal cargo and finally to disrupt normal operation of the company and ship systems.

# Cyber security and safety management

Cyber safety and security have equal potential to affect the safety of crew, vessel, and cargo. Cyber security is concerned with the protection of IT, OT and data from unauthorized access, manipulation and disruption.

Cyber safety is concerned with the risks from the loss of availability or integrity of safety critical data and OT. Cyber safety incidents can arise as the result of:

a.  A cyber security incident, which affects the availability and integrity of OT, for example corruption of chart data held in an Electronic Chart Display and Information System (ECDIS)

b.  A failure occurring during software maintenance and patching

c.  Loss of or manipulation of external sensor data, critical for the operation of a ship, which includes but is not limited to Global Navigation Satellite System.

# Management of network services and systems

In order to protect IT and OT on board systems while we have assess the cyber security risk, there is a need to implement both technical protection measures, safety network design and procedural measure as described below:

## Defense in depth

It is important to protect vital ship’s systems and data with multiple levels of protection measures which take into account the role of personnel, procedures and technology to increase:

(1). The probability that a cyber incident is detected.

(2). The effort and resources required to protect information, data and the availability of IT and OT systems.

The defense in depth comprise of:

(1).  Physical ship’s security according a security plan.

(2).   Network’s protection.

(3).   Intrusion detection.

(4).   Software signature library

(5).   Access and user controls

(6). Appropriate methods for using removable devices (passwords, encryption codes)

(7). Personnel’s awareness and training about the risk and the ways of handling a cyber incident.

## Use of administrator privileges

Access to significant or classified information should have only the authorized personnel. This can be achieved with the use of administrator privileges, which allow full access to system configuration settings and data. These privileges should only be given to trained and classified personnel, as well as in depend to their role and responsibilities in the company or onboard, to log into systems using such privileges. It is important that the use of administrator privileges should be only constrained to execution of functions requiring such access.

Another important thing is that the user accounts which are no more in use, they should be removed. Finally a robust password policy with strong encryption codes and passwords, which are changed periodically, should be established. Another significant issue is that the company policy about the frequency that a password should be changed and the size and the complexity of them should be addressed because difficult in use and memorized password can lead user to write them in a post-it in order to be easily accessible.

## Response and recovery plan Response plan

In order to have an effective response has first to identify the cyber security incident. After we have to define the objectives for response and investigate the situation so as to take appropriate actions to address the problem that affects our systems and then to recover systems, data and connectivity.

## Data recovery capability/ Back-up

When ship’s systems have been affected from a malicious attack, then the best solution to recover the systems is to restore data from a secure copy or image.  There should be essential information and software-adequate backup facilities so as to be available to appropriate personnel in case of a cyber incident. It is important to be a prioritize to the back-up procedures, for example it is vital to restore first OT systems, which are vital to the ship’s safe navigation and operation so as to regain navigational and operational capabilities quickly and safely after a cyber incident.

## Application software security (patch management)

Critical safety and security updates should be provided to onboard systems. These updates or patches should be applied correctly and in a timely manner to ensure that any flaws in a system are addressed before they are exploited by a cyber attack.

## Secure network design

Onboard networks should be partitioned by firewalls to create safe zones. The more firewalls that have to be passed through to access a zone, the more secure the systems and data in the zone. Confidential and safety critical systems should be in the most protected zone as illustrated at “Fig. 2”.

Fig. 2 Firewall safe zones

## Boundary defense

First of all a ship or company has to incorporate an Intrusion Detection System (IDS) system or an Intrusion Prevention System (IPS) so as there to be able to detect threats or malicious activity and in continue to log, report and block their activity. Moreover a baseline of network operations and expected data flows for users with defined roles and responsibilities has to be established and managed so as cyber incident alert thresholds can be established.

## Limitation to and control of network ports, protocols and services

Lists with authorized personnel, which can have access to network systems, can be used to implement the company’s security policy. This certifies that only appropriate traffic and use will be allowed through a controlled network or subnet. There should be as a requirement that routers useless ports should be closed to prevent unauthorized access to systems or data.

## Anti-virus and anti-malware tool updates

Procedural requirements and checklists should be implemented to ensure that updates are distributed to ships directors on a timely basis and that all relevant computers onboard are updated.

## Email and web browser protection

Email communication between the “ship to shore” path is a crucial part of a ship’s operation. Some methods for safe email exchange is to zip email or encrypted file, to disable hyperlinks on email system and to avoid use generic email addresses.

Appropriate email and web browser protection serves to:

a. Protect shore side and onboard personnel from potential social engineering

b. Prevent email being used as a method of hacking classified information

c. Ensure that the exchange of classified information through email is appropriately protected to ensure confidentiality and integrity of data.

d. Prevent web browsers and email clients from executing malicious scripts.

# Management plans

Company policies and procedures should ensure that cyber security is considered within the overall approach to safety and security risk management. It is significant the ship to conduct a security assessment so as to locate and identify all these procedures and operations that are needed to protect. The assessment should address communication systems, including computer systems and networks. This demand for controlling and monitoring the path from “ship to shore” of the Internet connection is so important owing to the fast adoption of sophisticated and digitalized onboard OT systems, that in many cases have not been designed to be cyber resilient.

The objective of the company’s Safety Management System is to provide a safe digital working environment by establishing appropriate instructions and procedures based on an assessment of all identified risks to the ship, onboard personnel and the environment. These instructions and procedures should consider risks causing from the use of IT and OT on board, as appropriate, taking into account applicable codes and recommended standards. Cyber risk management should:

a. Identify the responsibilities and duties of users, key personnel, and management both ashore and ship.

b. Identify the systems, assets and data, which if they will be violated, could pose risks to the ship’s operations and safety.

c. Implement technical protect measures against a cyber incident like configuration of networks, access control to networks and systems, communication defense and the use of malware and virus detection software.

d. Implement procedural protection measures to provide resilience against cyber incidents. This can be achieved with personnel training and awareness, software maintenance, use of access privileges and use of removable authorized media (flash disk with password).

e. Implement activities to prepare for and face cyber incidents.

# Conclusion

As we can see at “Fig. 3”, ships are becoming more and more integrated with shore’ side operations because digital communications are used in order to conduct business, manage operations, and stay in touch with head office of company.

Moreover, critical ship systems for the navigation, power and cargo management have been increasingly digitalized and connected to the Internet web to perform a wide variety of legitimate functions such as:

a. Engine performance monitoring

b. Maintenance and spare parts management

c. Cargo, crane and pump management

d. Voyage performance monitoring.

Fig. 3 “ships to shore” path and automation system

Between shore and ship stations are exchanged data, which may be of interest to cyber criminals to exploit. So, It is important that companies have to have full aware about the ship’s OT and IT systems and how these systems integrate with the shore side. This demands knowledge and knowhow of all computer based onboard systems and how safety, operations, and business can be compromised by a cyber incident.

Finally, it is not realistic to expect every ship to have experience to IT and cyber security but is a fact that most people today have some level of IT knowledge and security awareness through using their private computers so its easy for them to understand some procedures which are used to avoid malicious attacks. For example, crewmembers its critical to understand that an installation of a computer game in a computer, which is connected to the machine automation system could controls the ship’s automated power machines,  and consequently to increase the risk of a total blackout or a malicious attack. Moreover, making sure that a computer has an updated virus scanner before connecting it to the control network is a simple step.

# References

[1] Bagsvaerd, “The Guidelines on Cyber Security onboard Ships” https://www.marad.dot.gov/wpcontent/uploads/pdf/Guidelines_on_cyber_security_onboard_ships_version_1-1_Feb2016, retrieved Feb 1, 2016 [February 1, 2016].

[2] BIMCO, CLIA, ICS, INTERCARGO, INTERTANKO, OCIMF and IUMI, “The Guidelines on Cyber Security Onboard Ships” http://www.ics-shipping.org/docs/default source/resources/safety-security-and-operations/guidelines-on-cyber-security-onboard-ships.

[3] Hugh Boyes and Roy Isbell, “Code of practice, Cyber security for ships” https://maritimecyprus.files.wordpress.com/2017/09/cyber-security-code-of-practice-for-ships-2017.pdf, retrieved 2017.

[4] KAI HANSEN, AKILLUR RAHMAN, “Cyber threat to ships – real but manageable”https://library.e.abb.com/public/b9d267b4767c582f85257ca1003280e9/106_Cyber_threat_to_ships_real_but_manageable.pdf, retrieved January 2013.