Satellite Communications in Shipping

STATE OF ART

ANESTI Chrysafo

University of Thessaly

Informatics and Computational Biomedicine, ICB

Lamia, Hellas

canesti@gmail.com

BOCHORIS Konstantinos

University of Thessaly

Informatics and Computational Biomedicine, ICB

Lamia, Hellas

cobo69_98@gmail.com

Abstract—“Out of sight, out of mind” is probably one of the reasons we take satellites for granted, even though they play a crucial part in everything from TV broadcasting and transcontinental telephone calls to weather forecasting and the Internet.

Across the maritime industry, there is a growing trend toward IP-based applications and a need to stay connected with land-based operations. Commercial vessels require high-speed connections to run onboard business, maintain vessel management and maintenance applications as well as provide their crew Internet and VoIP connectivity to retain their talents.

For years, narrowband L-Band services with usage-based plans were the only communication solution onboard vessels forcing expensive tradeoffs and inadequate connectivity. In recent years, VSAT technologies more adequately addressed the demand for always-on broadband access and, as a result, have been adopted rapidly -- fueled by reduced antenna prices and sizes.

This paper takes a closer look at how satellite technology advances in the maritime industry.

Keywords—satellite; communication; shipping; maritime; Inmarsat; VSAT;)

# Introduction

In 2015 the global maritime satellite communications market reached close to 338,000 terminals, $953 million in revenue at the satellite operator level and $1.7 billion in revenue at the service provider level. The value of the maritime satcom market will continue to grow over the next decade, with a CAGR of 5% in terminals and revenue over the 10-year period. Ever-increasing data communications needs and the massive launch of new generation High Throughput Satellite (HTS) systems are expected to drive growth.

Demand for maritime satellite communications continued to grow driven by increasing operational communications needs, as opposed to crew communication years ago. Even though crew communications still represent the vast majority of a ship’s consumption, vessels’ operators are increasingly interested in big data-driven applications that improve operational efficiency. As a result, the number of VSAT terminals grew by 9% in 2015 to reach more than 16,000 commercial active terminals at year-end. A total of 8.5 Gbps of C-, Ku- and Ka-band capacity was used for maritime VSAT business, compared to less than 2Gbps in 2010.

2016 was an important milestone for maritime satellite communications. It marked the entry into service of HTS satellites with a major maritime focus, with the entry into commercial operation of Inmarsat Global Xpress and Telenor’s Thor7 satellite, in addition to the launch of the first two Epic satellites. A number of additional HTS satellites will be launched in the coming years, with maritime satellite capacity expected to reach ~680 Gbps by 2020. This will result in a significant decrease in capacity prices which, along with innovations at the antenna level with the entry into service of flat-panel antennas, will drive demand for more bandwidth-hungry applications such as big data analytics and video streaming applications.

A glance at maritime satellite communications evolution will smoothly introduce state-of-art of satellite communications in shipping

# Satellite Era in Shipping – Early  Days

As far as shipping is concerned, the satellite communications era began with the establishment in the 1970s of Inmarsat a not-for profit international organisation, set up at the behest of the International Maritime Organization (IMO) to provide a satellite communications network for the maritime community. Initially the service was used purely for commercial purposes allowing voice and telex communication with ships at sea equipped with an Inmarsat A terminal.

Without the advent of Global Maritime Distress and Safety System (GMDSS) and the mandatory requirement for most ships to be fitted with at least an Inmarsat C terminal, it is doubtful if the marine satellite communications sector would have expanded at anything like the rate it has. By having an Inmarsat terminal on board, ships immediately gained email as a new method of communication.

Early Inmarsat services were described by an alphabetical reference being Inmarsat A, B, C, D and E. When the letter F was reached, the service was renamed Fleet followed by a number (33, 55 or 77) indicating the size of the antenna in centimetres.

Inmarsat-A was the original Inmarsat service and offered analogue FM voice and telex services and, optionally, high speed data services at 56 or 64 kb/s. The service was withdrawn at the end of 2007.

Inmarsat-B provides voice services, telex services, medium speed fax/data services at 9.6 kb/s and high speed data services at 56, 64 or 128 kb/s.

Inmarsat-C effectively this is a “satellite telex” terminal with store-and-forward, polling etc. it can handle data and messages up to 32kb in length, transmitted in data packets in ship-to-shore, shore-to-ship and ship-to-ship direction. Message length for Inmarsat Mini C terminals may be smaller. Certain models of Inmarsat-C terminals with GPS are also approved for GMDSS use.

Inmarsat D/D+ is a paging service not regularly used on ships.

Inmarsat-E was a global maritime distress alerting service using small Emergency Position Indicating Radio Beacons (EPIRBs) that automatically relayed distress messages to maritime Rescue Coordination Centres. This service has been withdrawn.

Inmarsat Fleet 77 offers voice and the choice of mobile ISDN up to 64 kb/s or an always-on Mobile Packet Data Services (MPDS) for cost-effective, virtually global communications. Fleet 77 also meets the distress and safety specifications of the GMDSS for voice communication.

Inmarsat Fleet Broadband introduced in 2007 when the first of Inmarsat’s i-4 satellites went into service. It offer a shared-channel IP packet-switched service of up to 492 kb/s and a streaming-IP service from 32 up to X-Stream data rate. X-Stream delivers the fastest, on demand streaming data rates from a minimum of 384 kb/s up to around 450 kb/s. Most terminals also offer circuit switched Mobile ISDN services at 64 kb/s and even low speed (4.8 kb/s) voice services.

Inmarsat does not market its services directly but through service providers who are also able to market the services of other satellite network operators. Because of its GMDSS role, Inmarsat has dominated the marine satellite sector but it is not without competitors in the commercial communications arena. Among those competing, Iridium Communications was a frontrunner in voice and light data usage and had the advantage of being able to offer a wider pole to pole coverage that Inmarsat could not match due to the differences in the satellite constellations. Iridium has just begun a project under the title Iridium Next and is in the process of replacing its existing satellite network with a series of new satellites.

Since the advent of GMDSS, Inmarsat has become a private company and although committed to maintaining the safety services it is no longer focused solely on the marine sector as it once was. Recently the IMO has decided that the GMDSS communications should be opened up to other satellite operators. Iridium Communications announced it had formally submitted a comprehensive application to the IMO via the US government for the provision of mobile satellite communications in the GMDSS.

To put the horse before the truck, let’s examine first, what is a satellite and how does it work.

# Satellite “Modus Operandi”

A satellite doesn't necessarily have to be a tin can spinning through space. The word "satellite" is more general than that: it means a smaller, space-based object moving in a loop (an orbit) around a larger object. The Moon is a natural satellite of Earth, for example, because gravity locks it in orbit around our planet. The tin cans we think of as satellites are actually artificial (human-built) satellites that move in precisely calculated paths, circular or elliptical (oval), at various distances from Earth, usually well outside its atmosphere.

## Composition of a Satellite

Satellites are amazingly complex and expensive machines with tons of electronic bits and pieces jammed into them, but let's not get too bogged down in the details

Satellites are like any other vehicle inasmuch as they have two main parts: the generic vehicle itself and the specific thing it carries (the payload) to do its unique job.

: Basic Parts of Satellite

The "vehicle" part of a satellite is called the bus, and it includes the:

Outer case - This structural subsystem provides the mechanical base structure with adequate stiffness to withstand stress and vibrations experienced during launch. It maintains structural integrity and stability while satellite is in orbit, and shields the satellite from extreme temperature changes and micro-meteorite damage.

Power subsystem - Consists of Solar panels to convert solar energy into electrical power and batteries that store power and provide the satellite with power when it passes into the Earth's shadow.

Telemetry - A remote-controlled system that sends monitoring data from the satellite to Earth and operational commands back in the other direction.

Rocket thrusters - They keep the satellite in the correct orbital position, and

Reflective materials or other systems ("heat pipes") – This subsystem protects satellite from solar radiation and dissipates heat.

The payload might include transponders for a communications satellite, computers and atomic clocks to generate time signals for a navigation satellite, cameras and computers to images back to digital data for a photographic satellite, and so on.

We tend to group satellites either according to the jobs they do or the orbits they follow. These two things are, however, very closely related, because the job a satellite does usually determines both how far away from Earth it needs to be, how fast it has to move, and the orbit it has to follow.

## Satellite Orbits

One of the most surprising things about satellites is the very different paths they follow at very different heights above Earth. Left to its own devices, a satellite fired into space might fall back to Earth just like a stone tossed into the air. To stop that happening, satellites have to keep moving all the time so, even though the force of gravity is pulling on them, they never actually crash back to Earth. Some turn at the same rotational rate as Earth so they're effectively fixed in one position above our heads; others go much faster. Although there are many different types of satellite orbits, they come in three basic varieties, low, medium, and high—which are short, medium, and long distances above Earth, respectively.

- Low Earth Orbit (LEO) : Scientific satellites tend to be quite close to Earth—often just a few hundred kilometers up—and follow an almost circular path called a low-Earth orbit (LEO). Since they have to be moving very fast to overcome Earth's gravity, and they have a relatively small orbit (because they're so close), they cover large areas of the planet quite quickly and never stay over one part of Earth for more than a few minutes. Some follow what's called a polar orbit, passing over both the North and South poles in a "loop" taking just over an hour and a half to complete.

Medium Earth Orbit (MEO): The higher up a satellite is, the longer it spends over any one part of Earth. It's just the same as jet planes flying over your head: the slower they move through the sky, the higher up they are. A medium-Earth orbit (MEO) is about 10 times higher up than a LEO. GPS navstar satellites are in MEO orbits roughly 20,000 km (12,000 miles) above our heads and take 12 hours to "loop" the planet. Their orbits are semi-synchronous, which means that, while they're not always exactly in the same place above our heads, they pass above the same points on the equator at the same times each day.

- : Satellite orbits

- High Earth Orbit: Many satellites have orbits at a carefully chosen distance of about 36,000 km (22,000 miles) from the surface. This "magic" position ensures they take exactly one day to orbit Earth and always return to the same position above it, at the same time of day. A high-Earth orbit like this is called geosynchronous (because it's synchronized with Earth's rotation) or geostationary (GEO) (if the satellite stays over the same point on Earth all the time). Communications satellites—our "space mirrors"—are usually parked in geostationary orbits so their signals always reach the satellite dishes pointing up at them. Weather satellites often use geostationary orbits because they need to keep gathering cloud or rainfall images from the same broad part of Earth from hour to hour and day to day (unlike LEO scientific satellites, which gather data from many different places over a relatively short period of time, geostationary weather satellites gather their data from a smaller area over a longer period of time).

- : Satellite Orbits

| Satellite orbit definitions |  |  |  |
| --- | --- | --- | --- |
| Orbit name | Orbit initials | Orbit altitude (km above Earth's surface) | Details / comments |
| Low Earth Orbit | LEO | 200 - 1200 |  |
| Medium Earth Orbit | MEO | 1200 - 35790 |  |
| Geosynchronous Orbit | GSO | 35790 | Orbits once a day, but not necessarily in the same direction as the rotation of the Earth - not necessarily stationary |
| Geostationary Orbit | GEO | 35790 | Orbits once a day and moves in the same direction as the Earth and therefore appears stationary above the same point on the Earth's surface. Can only be above the Equator. |
| High Earth Orbit | HEO | Above 35790 |  |

There are a number of classifications associated with various different types of satellite orbits, for example:

- Based on distance from earth:

  - Circular satellite orbit:   For a circular orbit, the distance from the Earth remains the same at all times.

  - Elliptical satellite orbit:   The elliptical orbit changes the distance to the Earth

: Satellite orbits

- Based on direction of rotation around the Earth:

  - Posigrade: The rotation around the earth is said to be posigrade when it rotates in the same direction as the rotation of the Earth.

  - Retrograde: The rotation around the earth is said to be retrograde when it rotates in the opposite direction to the rotation of the Earth.

Among other types of satellites, communications satellites are of paramount importance for shipping due to the fact that ships are dependent on satellite communication when out of range of coastal communications systems.

# Communication Satellites

The story of communications satellite technology, however, began seriously in 1945. The overall vision of global satellite communications started in earnest with the writings of a young author and scientist released from service in the British Radar Establishment as the end of World War II drew near. In a privately circulated paper shared with fellow scientists in early 1945 and then in a formal article published in the fall of the same year, this young former air force officer outlined the future of satellite communications. In 1945, his article about "extraterrestrial radio relays" appeared in a now obscure British publication called Wireless World. Sir Arthur Clarke, at the time, was neither a knight of the realm, a world personality, nor a science fiction guru. Rather, he was a young Englishman with a strong yen for ocean diving and adventure. He went on to indulge these appetites with vigor in both Australia off the Great Barrier Reef and in Ceylon—now Sri Lanka—where he now lives. In the years that followed, he wrote an increasing array of science fiction novels and scientific papers, but none was ultimately as important as his early writings on satellites.

Communications satellites are "space mirrors" that can help us bounce radio, TV, Internet data, and other kinds of information from one side of Earth to the other. When used for communications, a satellite acts as a repeater. Its height above the Earth means that signals can be transmitted over distances that are very much greater than the line of sight. An earth station transmits the signal up to the satellite. This is called the up-link and is transmitted on one frequency. The satellite receives the signal and retransmits it on what is termed the down-link which is on another frequency.

## Uplinks and Downlinks

If you want to send something like a TV broadcast from one side of Earth to the other, there are three stages involved. First, there's the uplink, where data is beamed up to the satellite from a ground station on Earth. Next, the satellite processes the data using a number of onboard transponders (radio receivers, amplifiers, and transmitters). These boost the incoming signals and change their frequency, so incoming signals don't get confused with outgoing ones. Different transponders in the same satellite are used to handle different TV stations carried on different frequencies. Finally, there's the downlink, where data is sent back down to another ground station elsewhere on Earth.

: Uplink - Downlink

Although there's usually just a single uplink, there may be millions of downlinks, for example, if many people are receiving the same satellite TV signal at once. While a communications satellite might relay a signal between one sender and receiver (fired up into space and back down again, with one uplink and one downlink), satellite broadcasts typically involve one or more uplinks (for one or more TV channels) and multiple downlinks (to ground stations or individual satellite TV subscribers).

The circuitry in the satellite that acts as the receiver, frequency changer, and transmitter is called a transponder. This basically consists of a low noise amplifier, a frequency changer consisting a mixer and local oscillator, and then a high power amplifier. The filter on the input is used to make sure that any out of band signals such as the transponder output are reduced to acceptable levels so that the amplifier is not overloaded. Similarly the output from the amplifiers is filtered to make sure that spurious signals are reduced to acceptable levels. The signal is received and amplified to a suitable level. It is then applied to the mixer to change the frequency in the same way that occurs in a superheterodyne radio receiver. As a result the communications satellite receives in one band of frequencies and transmits in another.

In view of the fact that the receiver and transmitter are operating at the same time and in close proximity, care has to be taken in the design of the satellite that the transmitter does not interfere with the receiver. This might result from spurious signals arising from the transmitter, or the receiver may become de-sensitised by the strong signal being received from the transmitter. The filters already mentioned are used to reduce these effects.

: Block diagram of a basic Satellite transponder

Signals transmitted to satellites usually consist of a large number of signals multiplexed onto a main transmission. In this way one transmission from the ground can carry a large number of telephone circuits or even a number of television signals. This approach is operationally far more effective than having a large number of individual transmitters.

Obviously one satellite will be unable to carry all the traffic across the Atlantic. Further capacity can be achieved using several satellites on different bands, or by physically separating them apart from one another. In this way the beamwidth of the antenna can be used to distinguish between different satellites. Normally antennas with very high gains are used, and these have very narrow beamwidths, allowing satellites to be separated by just a few degrees.

## Frequency Allocation

A basic resource for making all kinds of wireless communications is the available spectrum of radio frequencies. For reasons of better management, the radio spectrum of frequencies is divided into bands with conventional names designated by the International Telecommunications Union.

- : IEEE Bands

| Frequency | IEEE bands | Use in telecommunications |
| --- | --- | --- |
| 3 - 300 Hz | ELF (Extremely Low Frequency | Communication with submarines |
| 30 - 300 Hz | SLF (Super Low Frequency) | Communication with submarines |
| 300 - 3000Hz | ULF (Ultra Low Frequency) | Submarine communication, communication within mines |
| 3 - 30 KHz | VLF (Very Low Frequency) | Navigation, time signals |
| 30 - 300 KHz | LF (Low Frequency) | AM longwave broadcasting |
| 300 KHz – 3 MHz | MF (Medium Frequency) | AM (medium-wave) broadcasts, amateur radio |
| 3 – 30 MHz | HF (High Frequency) | Shortwave broadcasts, citizens band radio |
| 30 – 300 MHz | VHF (Very High Frequency) | FM, television broadcasts, land mobile and maritime mobile communications, amateur radio, weather radio |
| 300 MHz – 1 GHz | UHF (Ultra High Frequency) | Television broadcasts, GSM |
| 1 – 2 GHz | L | Satellite maritime and mobile services, GSM |
| 2 – 4 GHz | S |  |
| 4 – 8 GHz | C | Communications satellites |
| 8 - 12 GHz | X |  |
| 12 – 18 GHz | Ku | Communications satellites, Direct Broadcast Satellite |
| 18 – 24 GHZ | K | Communication between LEO satellites and ground stations 19 - 20 GHz. Communication between LEO satellites 23 GHz |
| 24 – 40 GHz | Ka | Communication between ground stations and LEO satellites. The system is affected in weather conditions. |

## Satellite radio-communications categories

Fixed satellite service (FSS): A radiocommunication service between ground stations, in particular at given locations when one or more satellites are used. The Station position may be a specified fixed point or any fixed points within a specific region.

Broadcasting satellite service (BSS): A radiocommunication service in which signals transmitted or retransmitted by space stations are intended for direct reception by the general public. In the broadcasting-satellite service, the term “direct reception” shall encompass both individual reception and community reception.

Mobile satellite service (MSS): A radiocommunication service between mobile earth stations and one or more space stations, or between space stations used by this service; or between mobile earth stations by means of one or more space stations. There are three major types: AMSS (aeronautical MSS), LMSS (land MSS), and MMSS (maritime MSS).

Amateur satellite service (AmSS): A radiocommunication service for the purpose of self-training, intercommunication and technical investigations carried out by amateurs, that is, by duly authorized persons interested in radio technique solely with a personal aim and without pecuniary interest.

Radio determination service (RDSS): A radiocommunication service for the purpose of the determination of the position, velocity or other characteristics of an object, or the obtaining of information relating to these parameters involving the use of one or more space stations.

Standard frequency & time signal satellite service (SFSS):  A radiocommunication service for scientific, technical and other purposes, providing the transmission of specified frequencies, time signals, or both, of stated high precision, intended for general reception.

Earth exploration satellite service (EESS): A radiocommunication service between earth stations and one or more space stations, which may include links between space stations, in which information relating to the characteristics of the Earth and its natural phenomena, including data relating to the state of the environment, is obtained from active sensors or passive sensors on Earth satellites.

Space research service (SRS): A radiocommunication service in which spacecraft or other objects in space are used for scientific or technological research purposes.

Inter satellite service (ISS): A radiocommunication service providing links between satellites.

Space operation service (SpO): A radiocommunication service concerned exclusively with the operation of spacecraft, in particular space tracking, space telemetry and space telecommand. These functions will normally be provided within the service in which the space station is operating.

## Communications satellite advantages & disadvantages

Satellites are able to provide communications in many instances where other forms of communications technology may not provide a feasible alternative.

Communications satellites provide a number of advantages

- Flexibility: Satellite systems are able to provide communications in a variety of ways without the need to install new fixed assets.

- Mobility: Satellite communications are able to reach all areas of the globe dependent upon the type of satellite system in use, and the ground stations do not need to be in any one given location. For this reason, many ships use satellite communications.

- Speedy deployment:   Deployment of a satellite communications system can be very speedy. No ground infrastructure may be required as terrestrial lines, or wireless base stations are not needed. Therefore many remote areas, satellite communications systems provide an ideal solution.

- Provides coverage over the globe: Dependent upon the type of satellite communications system, and the orbits used, it is possible to provide complete global coverage. As a result, satellite communications systems are sued for providing communications capabilities in many remote areas where other technologies would not be viable.

When considering the use of a satellite some disadvantages also need to be taken into consideration.

- Maritime Communications

- Cost: Satellites are not cheap to build, place in orbit and then maintain. This means that the operational costs are high, and therefore the cost of renting or buying space on the satellite will also not be cheap.

- Propagation delay: As distances are very much greater than those involved with terrestrial systems, propagation delay can be an issue, especially for satellites using geostationary orbits. Here the round trip from the ground to the satellite and back can be of the order of a quarter of a second.

- Specialised satellite terminals required:   Even though the operator will operate the entire required infrastructure, the user will still need a specialised terminal that will communicate with the satellite. This is likely to be reasonably costly, and it will only be able to be used with one provider.

After this short journey to satellite - space it is time to dive into maritime issues.

# Maritime Communications

Radio telecommunication at sea had undergone a sea change in the last century. After the days of semaphores and flags (which is still relevant today in some cases), radio brought about a drastic change in marine communication at sea.

From the early years of the last century, ships started fitting radio for communicating distress signals among themselves and with the shore. Radio telegraphy using Morse code was used in the early part of the twentieth century for marine communication.

In the seventies, after considering the studies of the International Telecommunication Union, IMO brought about a system where ship-to-ship or ship-to-shore communication was put into action with some degree of automation, wherein a skilled radio officer keeping 24×7 watch was not required.

Marine communication between ships or with the shore was carried with the help of on board systems through shore stations and even satellites. While ship-to-ship communication was brought about by VHF radio, Digital Selective Calling (DSC) came up with digitally remote control commands to transmit or receive distress alert, urgent or safety calls, or routine priority messages. DSC controllers can now be integrated with the VHF radio as per Safety Of Life at Sea (SOLAS) convention.

The SOLAS Convention provides that “… every ship shall be provided with radio installations capable of complying with the functional requirements … throughout the intended voyage …”. Thus every ship has to carry a core installation of basic equipment that is applicable to all waters, supplemented by additional equipment that extends the ships communications capabilities according to the specific waters in which she will sail. These supplementary requirements are defined by the distance offshore the ship will travel.

Satellite services, as opposed to terrestrial communication systems, need the help of geo-stationary satellites for transmitting and receiving signals, where the range of shore stations cannot reach. These marine communication services are provided by INMARSAT (a commercial company) and COSPAS – SARSAT (a multi-national government funded agency).

While INMARSAT gives the scope of two way communications, the COSPAS – SARSAT has a system that is limited to reception of signals from emergency position and places with no facilities of two way marine communications, indicating radio beacons (EPIRB).

Naturally, the sea has become a lot safer with these gadgets and other important navigation tools recommended by the IMO and as enshrined in GMDSS.

There is a wide range of satellite communication systems available. Very roughly, they can be categorized as below

- : Satellite Communication Systems

| System | Band | Range | Bandwidth | Comment |
| --- | --- | --- | --- | --- |
| Inmarsat C | L | A3 | 9.6 kbps, packet oriented | GMDSS, Used for short e-mails and messages |
| Inmarsat Fleet 77/BGAN | L | A3 | 128-450 kbps | GMDSS (not BGAN yet), supports Internet |
| Iridium | L | A4 | 134 kbps (Open Port) | Also coverage in Arctic. |
| VSAT shared link | C, Ku, Ka | A1-A3 | Any, typical 64-512 kbps. Shared by several users. | Coverage varies with system, normally not deep sea. |
| VSAT dedicated link | C, Ku, Ka | A1-A3 | Any, dependent on price. Dedicated capacity to user. | Coverage varies with system and (high) price. |
| Other (Orbcomm, Globalstar, Thuraya, ARGOS) | L, S, C, Ku, Ka | A1-A4 | Typically low, usually up to telephone. | Either bent pipe systems or store and forward. |

## Maritime VSAT

Maritime VSAT is the use of satellite communication through a Very-Small-Aperture Terminal (VSAT) on a moving ship at sea. Since a ship at sea moves with the water, the antenna needs to be stabilized with reference to the horizon and True north, so that the antenna is constantly pointing at the satellite it uses to transmit and receive signals. Motors and sensors are used to keep the antenna pointed accurately at the satellite. This enables it to transmit to and receive from the satellite whilst minimising losses and interference with adjacent satellites. New technology is emerging which allows a solid state device (flat panel) to electronically steer the antenna without moving parts, this new technology is in its infancy but will be a major game changer for the industry.

### History

The Maritime VSAT (Business) Industry was first created for the US Navy in September 1986, by the joint venture team of Richard A. Hadsall, President and CEO of Crescomm Transmission Services and Robert J. Matthews, President of SeaTel Inc. The result of this business venture was the creation of the company Maritime Telecommunications Network, (MTN) Inc. MTN went on to commercialize the Maritime VSAT business by delivering services to Various Cruise Lines around the world as well as many Commercial Oil and Gas installations and vessels. MTN also pushed for global Maritime VSAT recognition with the petition for rule making from the FCC and the lobbying of the ITU’s World Radio Commission (WRC) for recognition to use the fixed satellite service (FSS) in C and Ku band by creating new rulings to recognize Earth Stations on Vessels ESV. This was accomplished by revision of the Radio Regulations, complementing the Constitution and the Convention of the International Telecommunication Union ITU, which incorporated the decisions of the World Radio Communication Conferences of 2003 (WRC-03).

### Technology

Initially, maritime VSAT was using single channel per carrier technology, which suited large-volume users like oil drilling rigs and oil platforms and large fleets of ships from one shipowner sailing within one or few satellite footprints. This changed when the company iDirect launched its IP-based time-division multiple access technology that dynamically allocated bandwidth to each ship for shared bandwidth, lowering the entry-level cost for getting maritime VSAT installed, which turned out to be of key importance to small to mid-sized fleets, and thus to the market acceptance of VSAT.

There are many different options to build a maritime broadband network onboard of a vessel. Each option has its advantage (and disadvantage) in cost, in the coverage, the signal strength and requirements for the antenna size (and thus the requirements of installation).

There are major differences in capabilities, features, cost and performance between VSAT (Geostationary orbit satellites in Ku-band, C-band and Ka-band) and Low Earth orbit or Medium Earth Orbit satellites with L-band technologies in use.

Both L-band (LEO & MEO) and VSAT (GEO) systems are marketed with what appear to be a shared set of features and benefits.

Existing systems VSAT is a larger type of terminal operating towards geosynchronous satellites on C-, Ku- and Ka-bands. The majority of VSAT antennae range from 60 cm to 1.2 m. Given the large amounts of available spectrum on these bands, data rates are considerably higher and traffic costs significantly lower than L-band systems like Inmarsat and Iridium.

### Market

Typical VSAT offerings can provide data speeds of up to 6 Mbit/s, and the typical subscription will involve a monthly lease with a cap on a certain data volume. The subscription fee will depend on the satellite dish size, coverage, and bandwidth – and also whether the data rates are provided on a guaranteed or best effort basis. A typical VSAT subscription costs between $1000 and $5000, with incremental traffic costs at <$1/per MB.

In contrast to MSS at L-band, the VSAT market is highly competitive and has been an active arena for many commercial and technological developments in recent years. VSAT service providers typically bundle packages with modem, antenna, and a satellite resource lease from a satellite operator such as Eutelsat, Intelsat, or Telenor. Satellites delivering VSAT services are typically in geostationary orbit, implying that service quality deteriorates beyond 70º latitude. However, with clever antenna installations and link budget margins, it is possible to offer acceptable service quality as far as 75-78º North.

Due to the lower antenna gain, VSAT services on C-band provide excellent deep-sea coverage due to wide beams covering large sea areas, but also require larger ship antennae. Over the last years Kuband coverage has improved, and can now offer a very cost-effective alternative to C-Band. On Ka-band there are still only a few satellites, but due to the amount of bandwidth available, and the potential efficiency benefits from the higher antenna gain, the prices have the potential to drop below those of Ku in the longer term.

According to the Maritime VSAT report issued by the Comsys Group, revenues from stabilised VSAT services (which include bundled hardware) grew to over $1.3 billion in 2014 and the number of vessels, including rigs and platforms, in service closed in on 21,000.

### Barriers

The main barrier for the adoption of VSAT is the size and cost of the end-user equipment. Due to advanced terminal electronics and the large stabilized antenna platform, equipment and installation costs for a maritime VSAT installation are typically in the range $25-$50k, which is much higher than L-band products. So far only heavy data users have found this initial investment worthwhile, which explains why there are considerably fewer VSAT installations than smaller, cheaper, and more narrow-band L-band products.

Although VSAT is much better than MSS L-band in terms of cost per bit and available bandwidth, Ku-band allocations have been filling up, and there are many on-going regulatory fights for spectrum. So cost and bandwidth are still bottlenecks, making it expensive to accommodate bandwidth hungry applications in a maritime setting.

Regarding rules, regulations, and standards for functionality and performance related to VSAT, there is a vacuum; virtually nothing exists in the public domain. Statutory regulations concerning GMDSS and navigational safety simply do not recognize VSAT as a communication option. For emerging nonsafety related connectivity applications there is also a need to document best practices and standardize interfaces to ensure effective, future-proof implementations and to achieve vendor competition and economies of scale.

Another challenge for VSAT is the susceptibility to rain fade due to the absorption of microwave radio frequency signals by atmospheric rain, snow, or ice. This effect is significant above 11 GHz, and hence especially challenging for Ka-band. Operators need to plan in link margins or other mitigations in order to avoid reduced service reliability.

## Other Mobile Satellite System (MSS) on L Band

### INMARSAT

Since its founding in 1976, Inmarsat has been a reliable provider of satellite communications services to the maritime community. Inmarsat owns and operates a network of L-band satellites in Geostationary orbit (35,786 km altitude) and gateways connected to public networks. Inmarsat’s system design has traditionally been in-house and they have created their own proprietary products, generating traffic on their satellites. Inmarsat takes an active role in operations, distribution, and sales, but still considers itself as a “wholesaler” based on a wide international network of distribution partners.

Inmarsat provides a wide range of maritime services over their L-band satellites. Inmarsat-C is a low-rate messaging product that was launched in 1992, and that typically forms part of the equipment set-up for GMDSS vessels. Fleet Broadband is a family of broadband products launched in 2007, designed for the Inmarsat-4 satellites and providing 3G-type services up to 500 kbps.

The strengths of the current Inmarsat products are reliability, coverage, and a streamlined way of delivering services. Inmarsat is the dominant player, with ~90 % market share within mobile satellite systems (L-band).

### IRIDIUM

Like Inmarsat, Iridium is an L-band satellite operator, but Iridium’s satellites are in Low Earth Orbit (LEO), at approximately 780 km altitude. Iridium became operational in 1998 with a constellation of 66 satellites. The Iridium satellites can communicate directly with neighbouring satellites via Inter-satellite links (ISL), thus reducing the need for gateways and on-ground switching. The satellites have an orbital speed of 27000 km/s, use 100 minutes to orbit the earth, and approximately 8 minutes to cross the horizon as seen from a fixed location on the ground. This creates system challenges such as Doppler effects, and satellite handovers during user sessions. Iridium’s key strength is that the constellation of constantly moving LEO satellites provides continuous coverage over the entire surface of the Earth, including Polar regions.

### THURAYA

Thuraya is a regional L-band system using two Geo-satellites, providing maritime products such as OrionIP, with IP data capabilities that are similar to those of Inmarsat’s Fleet-Broadband.

### ORBCOMM

OrbComm operates a satellite network of 29 LEO satellites at 775 km altitude, optimized to support M2M (machine-to-machine) applications. Unlike the other MSS providers operating in L-band, OrbComm is the only licensee operating in 137-150 MHz VHF band, which was allocated globally for “Little LEO” systems.

### Barriers

The MSS satellite communications market suffers from poor competition, with Inmarsat as the dominant player, partly based on its unique position in GMDSS.

Given the market situation and the relatively little bandwidth available for MSS systems on L-band, user data rates are relatively limited and prices are quite high. Inmarsat and Iridium have done a good job in making products that provide a few hundred of kbps, but at high charges and with comparatively limited throughputs compared with VSAT systems and terrestrial options.

Regulations and standards are trailing behind technological and commercial developments. The only MSS product with any substantial status within GMDSS is Inmarsat-C. This was launched in 1992 and is limited to messaging applications. Regulators have not yet utilized MSS products’ broadband capabilities to enhance safety, navigation, and to protect the environment. Despite unique Polar coverage and success with voice and M2M tracking applications, Iridium has taken 17 years to become considered for inclusion as a GMDSS operator.

Finally, lack of available spectrum on L-band is a barrier that will constrain further growth of these systems and, as a result, MSS operators like Inmarsat and Iridium are moving to higher frequency bands for their new systems.

New systems and innovations due to the lack of spectrum, new large-scale investments or future deployments in broadband capabilities at L-band should not be expected. L-band MSS products will still have an important role in the future, but for any new developments the emphasis is likely to be on narrow-band messaging and tracking applications with smaller terminals, while higher frequency bands (VSAT Ku and Ka) will be the main arena for broadband developments.

## Global Maritime Distress and Safety System (GMDSS)

The Global Maritime Distress and Safety System is the technical, operational and administrative structure for maritime distress and safety communications worldwide. It was established in 1988 by the IMO which adopted a revised text of Chapter IV of the International Convention for the SOLAS - dealing with Radiocommunications - and was implemented globally between 1992 and 1997. The GMDSS establishes the radiocommunications equipment that ships are required to carry, how this equipment shall be maintained and how it is used, and provides the context within which governments should establish the appropriate shore-based facilities to support GMDSS communications.

### Basic concept of the GMDSS

In the days since the very first radio equipment was used at sea, most famously in the sending of a distress message from the Titanic, vessels in distress relied almost exclusively on their ability to alert other ships in order to obtain assistance. The GMDSS, for the first time, changed this procedure and established a new fundamental principle that a ship in distress should send its alert to a shore, which would then accept the responsibility of co-ordinating the necessary rescue efforts. Thus the GMDSS became inextricably linked with the parallel implementation of the International Search and Rescue Convention (SAR Convention) and the development of shore facilities within the structure of the World-Wide SAR Plan.

In addition to improving the capability of ships to declare their distress and receive assistance co-ordinated from the shore, the GMDSS also provided for the broadcast of essential safety-related information – Maritime Safety Information (MSI) – which could be received automatically on board ships at sea and would offer ships the chance to navigate more safely on a routine basis.

### Application

The GMDSS generally applies to all ships over 300 gross tons and upwards on international voyages. However, GMDSS systems are equally valuable for other vessels, including recreational and other “voluntary fit” vessels. GMDSS carriage requirements are also applied to some fishing vessels by national legislation.

### Functional Requirements

The GMDSS therefore provides that every ship, while at sea, shall be able to perform the following nine basic communication functions:

- transmitting ship-to-shore distress alerts (by at least two separate and independent methods);

- receiving shore-to-ship distress alerts;

- transmitting and receiving ship-to-ship distress alerts;

- transmitting and receiving search and rescue co-ordinating communications;

- transmitting and receiving on-scene communications;

- transmitting and receiving signals for locating;

- transmitting and receiving maritime safety information;

- transmitting and receiving general communications; and

- transmitting and receiving bridge-to-bridge communications.

### Ship Requirements

For international operational requirements, GMDSS has divided the world in four sub areas. These are four geographical divisions named as A1, A2, A3 and A4. Different radio communication systems are required by the vessel to be carried on board ships, depending on the area of operation of that particular vessel:

- A1 – It’s about 20- 30 nautical miles from the coast, which is under coverage of at least one VHF coast radio station in which continuous DSC alerting is available. Equipment used: A VHF, a DSC and a NAVTEX receiver (a navigational telex for receiving maritime and meteorological information).

- A2 – This area notionally should cover 400 nautical miles off shore but in practice it extends up to 100 nautical miles off shore but this should exclude A1 areas. Equipment used: A DSC, and radio telephone (MF radio range) plus the equipment required for A1 areas.

- A3 – This is the area excluding the A1 & A2 areas. But the coverage is within 70 degrees north and 70 degree south latitude and is within INMARSAT geostationary satellite range, where continuous alerting is available. Equipment used: A high frequency radio and/ or INMARSAT, a system of receiving Maritime Safety Information (MSI) plus the other remaining systems for A1 and A2 areas.

- A4 – These are the areas outside sea areas of A1, A2 and A3. These are essentially the Polar Regions North and South of 70 degree of latitude. Equipment used: HF radio service plus those required for other areas.

All oceans are covered by HF marine communication services for which the IMO requires to have two coast stations per ocean region. Today almost all ships are fitted with satellite terminal for Ship Security Alerts System (SSAS) and for long range identification and tracking as per SOLAS requirements.

In addition to the general communications equipment they are required to fit, ships also have to carry equipment for primary distress alerting – an Emergency Position Indicating Radio Beacon (EPIRB) and for the receipt of Maritime Safety Information (MSI).

EPIRBs are small floating (some can automatically float free from a sinking ship) buoys and work through the COSPAS-SARSAT 406MHz satellite system. They can send a distress alert to the shore automatically to alert a Rescue Co-ordination Centre (RCC) that a ship is in distress in a particular location, but they cannot be used for two-way voice or data communications.

MSI is a broadcast of essential safety-related information (navigational warnings, meteorological warnings and forecasts, and other vital information) from the shore to all ships. The shore authorities co-operate to structure this broadcast in such a way that intelligent receivers can discriminate between information that is relevant to a particular ship and other information that is not, automatically discarding those messages that are not relevant to the ship in which the particular receiver is carried. This, of course, requires some specific user choices to be applied in the software before the equipment can operate in this way. There are two systems a ship can use to receive MSI: NAVTEX and SafetyNET:

- NAVTEX is a terrestrial MF system, used to make broadcasts up to around 450 miles from the transmitter. Broadcasts are co-ordinated in time, coverage and content so that ships can limit what they receive to only the specific information relevant to the waters in which they are operating. NAVTEX transmissions are provided for most of the highly populated coastal waters of the globe;

- SafetyNET is a satellite-based system that is used to broadcast MSI to all other waters of the globe, except Sea Area A4. SafetyNET is a service provided via the Inmarsat C system, and is used to address MSI to the 23 NAV/MET Areas of the world. A ship’s terminal will normally select automatically the area in which she is currently located, and the ship’s staff have the option to receive also information addressed to the area in to which she is next heading.

Information broadcast on NAVTEX is not normally also broadcast via SafetyNET.

The SOLAS Convention also places responsibilities on ships to maintain watches on specific frequencies for distress and safety communications, so some element of ship-to-ship alerting is retained in the new system; and specifies in general terms to means to be employed to preserve the operation of the radio equipment through reserve sources of electrical supply. It deals also with the IMO Performance Standards – IMO’s series of operational criteria that each type of equipment must meet; plus how the equipment shall be maintained and the basic qualifications needed to operate the equipment safely and reliably.

### Undertakings by Governments

While the requirements for ships are detailed and explicit, governments cannot be held by international regulation to provide particular shore facilities in quite the same way. The GMDSS therefore includes a provision whereby each government that chooses to sign the SOLAS Convention (called a Contracting Government) “… undertakes to make available, as it deems practical and necessary either individually or in co-operation with other Contracting Governments, appropriate shore-based facilities for space and terrestrial communications …”.

This undertaking establishes one of the really significant features of the GMDSS infrastructure: some communication systems will use facilities that are essentially international in nature and not under the control or supervision of any single government. It is this that gives rise directly to the need for the international community to establish special arrangements for the international supervision of satellite-based communication facilities for the GMDSS, which have been developed and are implemented by International Mobile Satellite Organization (IMSO).

# trends in Maritime Satellite Communications

In April 2015, Futurenautics Research, a leading provider of maritime research and insight undertake a survey of ship operators and crew within the maritime market. The objective of the survey was to understand the satellite service implications arising from the maritime market’s current and future deployment of software applications at sea and ashore.

The key findings of the survey are:

## Distribution of Satellite Communications Systems

- Inmarsat FleetBroadband was the most commonly fitted satellite communication solution across respondents fleets. 82% of the fleets within the survey had Inmarsat FleetBroadband terminals fitted.

- VSAT solutions were present in 57% of fleets now making them the second most common satellite solution fitted in the commercial maritime sector.

- Only VSAT solutions were exclusively used 100% of the time as the primary data solution on-board.

- The average expenditure on L-Band solutions ranged from $50 to $3,500 USD/month. The majority of expenditure was in the range $0-$250 USD/month for those back-up systems such as Iridium and Thuraya.

- Inmarsat Fleet and FleetBroadband expenditure was concentrated around the $750-$1,500 USD/month range.

- Average expenditure for VSAT varied according to the type of solution fi tted. Regional/mini-VSAT installations were in the range of $750-$1,000/vessel/month.

- Typically, Ku-Band system expenditure was around the $2,750-$3,500/vessel/month range – with just over half the fleets spending in this range.

- All VSAT units were used as the primary data device both deep sea and in port/coastal waters.

## Reasons for choosing existing satellite solutions

- Operational efficiency is the most important reason for choosing a particular communications solution—for the first time more important than cost reduction. It potentially marks a very important shift in mind-set from a focus on fitting solutions to reduce cost, to understanding that they can deliver value across the business and drive competitive advantage.

- Crew welfare, so frequently cited in the past as a primary reason for fitting VSAT, was only chosen by 3% of respondents as the most important reason for fitting.

- This is in stark contrast to the requirements of crew. 72% of crew thought that the level of connectivity provided on board was a factor in choosing which ship operator they worked for. Of that 72% of respondents 78% said that it was a strong, or very strong influence on which contract they decided to take.

## Data Growth

On average ship operators believe that ship-toshore data traffi c would increase by nearly 60% over the next 2-3 years.

## Satellite solution upgrade path

- Respondents overwhelmingly thought that VSAT and next generation VSAT solutions would be the most suitable solution for their fleet’s future data requirements. When asked which solution they would upgrade to next and for which vessel type, only 18% of respondents choose an L-Band solution.

- There has been a significant shift in perception and acceptance of VSAT and HTS services by all sectors of the maritime market.

## Future Crew requirements

- On average, crew take 3 devices on-board. The most popular devices are Smartphones, laptops and external hard drives.

- Internet access is still the most demanded service with 73% of respondents believing that adequate crew Internet service could only be achieved with bandwidth of 512kbps or above, putting it well outside existing L-Band capabilities.

- The future service most wanted by crew was free in-port WiFi.

# Future in Maritime Satellite communications

It is tricky to predict the future, especially in the maritime communications – what are the future demands? Which technologies will be available? How will the regulatory bodies think in the future? Of course surveys give us a good sense of trend. Next step is to examine the new systems and innovations in the satellite industry.

## Inmarsat Global Xpress

Inmarsat’s 5th generation of satellites (three I-5 satellites launched in 2013-2015) will support a new network called Global Xpress (GX), with a global network entering commercial service early in the second half of 2015. With Ka-band satellites built by Boeing, Inmarsat are entering the VSAT world with a new maritime service called Fleet Xpress. The Fleet Xpress service includes a FleetBroadband terminal together with 1m or 60 cm GX Ka-band parabola, that, according to Inmarsat, is capable of delivering data rates of up to 5 Mbps/50 MBps (uplink/downlink) over all maritime trading routes. Inmarsat anticipates equipment pricing for GX to be in line with that for equivalent Ku-band VSAT devices. The system design has been delivered to Inmarsat by iDirect, who also delivers a “Core Module” that may be licensed to other manufacturers for incorporation in their products. Rain fading is a well-known problem for Ka-band, but the Fleet Xpress system has built-in adaptive code modulation so that the data rate can change with variable signal-tonoise levels. The GX system is designed for a target availability of 99 %, but the experienced availability should be higher as FleetBroadband (L-band) may be used as backup.

## Iridium Next

Iridium’s current satellite segment is approaching end-of-life, and Iridium will be launching new satellites from mid 2015 onwards.  Like Iridium’s current satellite constellation, Iridium NEXT will deploy a cross-linked mesh architecture with 66 LEO satellites providing continuous global coverage, including the Polar regions. A new feature with the Iridium Next satellites is that in addition to L-band transponder to support existing products, it will also contain a Ka-Band transponder to support new broadband services.

In February 2015, Iridium announced their plans to launch Certus, a new broadband service on the NEXT satellites delivering data rates of up to 1.4 Mbps, an impressive 10-fold increase on Iridium Openport. Iridium have partnered with experienced vendors, including Cobham Satcom and Rockwell Collins, to provide the new broadband offering.

## Intelsat EpicNG

Intelsat’s new high-throughput satellites initiative is called Intelsat EpicNG. The new satellites will have C-, Ku-, and Ka-transponders, complementing their existing fleet of around 50 C- and Ku-band satellites. The two first EpicNG satellites, Intelsat 29e and Intelsat 33e, are in-service from 2016. According to Intelsat, the new EpicNG platform will provide 3-5 times more capacity per satellite than their traditional fleet. The expected throughput of the satellites will vary according to application and satellite, but is expected to be in the range of 25-60 Gbps, typically 10 times higher than the traditional fleet.

As Intelsat is an existing VSAT satellite operator on C- and Ku-band, it is in the position to make complementary service offerings by combining C-, Ku- and Ka-band from existing and new satellites, in order to utilize the strengths and mitigate the weaknesses of each band. In many ways, Intelsat’s investment in the new EpicNG satellites can be considered a low-risk, organic extension of their existing operations.

While Inmarsat are designing their Fleet Xpress product with a closed group of partners under contract, Intelsat takes a more “open” and collaborative approach, allowing customers to design product offerings that exploit the new satellite capabilities, such as mesh networks, with guaranteed data rates and special service attributes.

## O3b

The O3b Satellite Constellation is a new Ka-Band system that started operation in September 2014. Eight satellites have been launched. The satellites are in Medium Earth Orbit (MEO) at 8063 km altitude, and each is equipped with 12 fully steerable Ka-band antennae, using a total of 4.3 GHz of spectrum corresponding to a throughput of 12 Gbps per satellite. According to O3b, their system is low-latency (130 ms round trip) and connectivity speeds are over 500 Mbit/s, making it a potent communication alternative. O3Bs approach for maritime is to track vessels with steerable spot beams, providing the tracked vessel with data rates up to 350 Mbps. The scalability of this approach has been questioned as O3B’s satellites have only 12 steerable spot beams, but may prove viable for larger cruise vessels with huge communication needs. O3B’s service coverage is limited beyond +/- 45º latitude, and there is no service beyond +/- 62º.

In general, there are a number of on-going HTS (High Throughput Satellites) initiatives, but many are focused on land-based markets and fixed installations for TV broadcast distribution. However, some HTS will also have return transponders to provide interactive services over maritime regions. One example is Telenor Satellite Broadcasting’s new HTS “Thor VII” that will provide maritime VSAT connectivity across the North Sea, Red Sea, Baltic Sea, and the Mediterranean, as well as the North Atlantic. Thor VII was launched in April 2015 and will offer 6-9 Gbps throughput, with up to 25 simultaneously active spot beams. According to Telenor, service offerings will include download speeds in the tens of Mbps and uplink speeds in the range 2-6 Mbps (depending upon antenna size).

Other examples of new HTS systems providing maritime coverage include ViaSat2, covering large parts of the North Atlantic, and Telesat’s Vantage 12 that was launched in late 2015 to provide coverage to high traffic maritime zones in the Mediterranean, North Sea, Caribbean, and South Atlantic.

# Conclusion

## Take Away

I imagine I’m not in the minority here when I say that the world of satellite communications is a bit confusing. Looking around at marine trade shows I see brands like Inmarsat, Iridium, Intelsat interspersed with terms like FleetBroadband and VSAT, and by the way, Inmarsat seems to have bizarre and confusing relationships with most of these satellite communications companies.

There are two main types of marine satellite services, you have MSS and VSAT.  MSS stands for Mobile Satellite Services, and VSAT stands for Very Small Aperture Terminal.  The difference, if you think of it in terms of mobile phone contracts, MSS is pay as you go, smaller equipment, smaller domes.  Inmarsat and Iridium both offer MSS.  VSAT uses a larger dome, requires a bit more significant commitment when it comes to installation, but it’s like having a mobile phone contract where you pay a fixed monthly contract for unlimited use.

The market is reflecting a rapid shift from MSS to VSAT operating specifically in C-Band, Ka-Band, and Ku-Band. This technology shift is attributed to bandwidth and cost advantages that VSAT offers to marine users.

## Outcome

Whether you are part of an offshore oil & gas operations team, a petroleum tanker fleet, transport or fishing vessels, or cruise line operator, maritime satellite communications can help you with a variety of telephone and data communications solutions to improve life at sea.

Satellite communication technologies transform life at sea by enabling corporations to track their ships and cargo across the globe; while the crew can stay in touch with both family and business colleagues to be informed of changes in real time; keeping the business of shipping, fishing or transport closer in touch with the rest of the world.

##### References

- COMSYS. “The Comsys Maritime VSAT Report,” n.d. http://www.comsys.co.uk.

- Euroconsult. “PROSPECTS FOR MARITIME SATELLITE COMMUNICATIONS.” Executive Report, 2016.

- Futurenautics Ltd. “Maritime Satellite Communications & Applications 2016.” London, 2016. http://www.futurenautics.com.

- iDirect. “Maritime VSAT Communications Solutions.” Herndon, June 2010. http://www.idirect.net.

- “International Convention for the Safety of Life at Sea.” International Maritime Organization, 1974.

- Latarche, Malcolm. “The Basics of Satellite Technology on Ships.” Blog. ShipInsight, June 13, 2017. https://shipinsight.com/the-basics-of-satellite-technology-on-ships.

- Maral, Gérard, Michel Bousquet, and Zhili Sun. Satellite Communications Systems: Systems, Techniques and Technology. Wiley, 2011.

- Pelton, Joseph, Robert Oslund, and Peter Marshall. Communications Satellites Change Agents. LAWRENCE ERLBAUM ASSOCIATES, 2004.

- Penttinen, Jyrki, ed. The Telecommunications Handbook Engineering Guidelines for Fixed, Mobile and Satellite Systems, n.d.

Euroconsult. “PROSPECTS FOR MARITIME SATELLITE COMMUNICATIONS.” Executive Report, 2016.

Latarche, Malcolm. “The Basics of Satellite Technology on Ships.” Blog. ShipInsight, June 13, 2017. https://shipinsight.com/the-basics-of-satellite-technology-on-ships/.

Pelton, Joseph, Robert Oslund, and Peter Marshall.Communications Satellites Change Agents. LAWRENCE ERLBAUM ASSOCIATES, 2004.

Range code will be explained in GMDSS chapter

COMSYS. “The Comsys Maritime VSAT Report,” n.d. www.comsys.co.uk.

Futurenautics Ltd. “Maritime Satellite Communications & Applications 2016.” London, 2016.www.futurenautics.com.